Stopping and Falling Worksheet 
This worksheet accompanies Stopping Distances.ppt and Falling Objects.ppt

1a.  What is meant by the phrase ‘thinking distance’ when applied to vehicles?

……………………………………………………………………………………………..…………………………………………………………………………………………………………..………

b. Describe two factors that might increase thinking distance.

…………………………………………………………………………………………………….……..…………….........................................................................................................................

2a.  Use the following data to draw a line graph in the space below:

| speed (mph) | 20 | 30 | 40 | 50 | 60 | 70 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| thinking distance (m) | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| braking distance (m) | 6 | 14 | 24 | 38 | 54 | 75 | 96 |

b. What happens to the thinking distance when the speed doubles?

………………………………………………………………………………………………………………………………………………………………………………………………………………

c. Describe two factors that might affect the braking distance of a vehicle.

……………………………………………………………………………………………………...………………………………………………………………………………………………..……

- Fill in the gaps.

Speed and braking distance increase in a ……………………… relationship. For example,

if speed increases by a factor of 3, braking distance increases by a factor of 32 (9).

……………………… is a factor that affects both thinking and braking distance.

The stopping distance of a vehicle is ……………………………………………………………….. …………………………………………………………………………………………………………………………………………………………………………………………………………………………

4.  Calculate the stopping distance for a car travelling at 50 mph with a 15 m thinking distance and a 30 m braking distance. Show your working.

………………………………………………………………………………………………………………………………………………………………………………………………………………………...

…………………………………………………………………………………………………………...

…………………………………………………………………………………………………………...

…………………………………………………………………………………………………………...

…………………………………………………………………………………………………………...

- 5.  When brakes are applied in a car, the work done by the frictional forces between the brake pads and brake discs cause the kinetic energy of the car to be converted into heat energy.

- Use the following equation to calculate how much work is done by the brakes to stop a 2,050 kg jeep travelling at 50 m/s.

- work done = energy transferred

- work done = ½  ×  mass  ×  velocity2

- …………………………………………………………………………………………………………….…………………………………………………………………………………………………….…………………………………………………………………………………………………………………

- 6.  A student carried out an experiment to measure the effect of air resistance on several objects which he dropped from a height of 4m:

| - objects dropped | - time to fall to the ground (s) |
| --- | --- |
| - sheet of A4 paper | - 3.6 |
| - half a sheet of scrunched-up A4 paper | - 2.1 |
| - penny | - 1.3 |
| - opened up-plastic bag | - 4.8 |

- a.  Why did the penny reach the ground before the plastic bag?

- ………………………………………………………………………………………………………

- ………………………………………………………………………………………………………

- ………………………………………………………………………………………………………

- ………………………………………………………………………………………………………

- b. Which object do you think would accelerate the fastest towards the ground

- when dropped? Why?

- ……………………………………………………………………………………………………….…………………………………………………………………………………………………………

- …………………………………………………………………………………………………………

- c. Explain how and why the results would be different if the same objects were dropped from the same height on the Moon.

- …………………………………………………………………………………………………………

- …………………………………………………………………………………………………………

- …………………………………………………………………………………………………………

- ……………………………………………………………………………………………………………………………………………………………………………………………………………………

- 8.  The weight of an object depends on its mass and the gravitational field strength.

- Use the following equation to work out the mass of a person with a weight of 380 N and gravitational field strength of 10 N/kg. Show your working.

weight = mass × gravitational field strength

- ………………………………………………………………………………………………………………………………………………………………………………………………………………………..…………………………………………………………………………………………………………..

- …………………………………………………………………………………………………………..

- …………………………………………………………………………………………………………..

- …………………………………………………………………………………………………………..

- …………………………………………………………………………………………………………..